# Part1

code;

import devsim

device = "Resistor"

region = "Silicon"

# --- 1. Mesh Creation ---

# 1D mesh (0 to 50um) with 100 node points

# Spacing (ps) = 50e-4 / 100 = 5e-7

devsim.create_1d_mesh(mesh="mesh1")

devsim.add_1d_mesh_line(mesh="mesh1", pos=0.0, ps=5e-6, tag="top")

devsim.add_1d_mesh_line(mesh="mesh1", pos=50e-4, ps=5e-6, tag="bot")

devsim.add_1d_contact(mesh="mesh1", name="top", tag="top", material="metal")

devsim.add_1d_contact(mesh="mesh1", name="bot", tag="bot", material="metal")

devsim.add_1d_region(mesh="mesh1", material="Si", region=region, tag1="top", tag2="bot")

devsim.finalize_mesh(mesh="mesh1")

devsim.create_device(mesh="mesh1", device=device)

# --- 2. Parameters ---

ni = 1.0e10

Vt = 0.02585

c1 = 1.0e17        # Magnitude of Donor concentration

c2 = 0.0           # Magnitude of Acceptor concentration (per prompt example)

devsim.set_parameter(device=device, region=region, name="ni", value=ni)

devsim.set_parameter(device=device, region=region, name="Vt", value=Vt)

# --- 3. Net Doping Profile ---

# Following your prompt: Net doping = (c1 * Ndn) - (c2 * Nap)

# For a uniform resistor (Fonstad 6.2), Ndn = 1 across the whole device

devsim.node_model(device=device, region=region, name="NDn", equation=f"{c1}")

devsim.node_model(device=device, region=region, name="NAp", equation=f"{c2}")

devsim.node_model(device=device, region=region, name="NetDoping", equation="NDn - NAp")

# --- 4. Potential Solution (Zero Bias) ---

devsim.node_solution(device=device, region=region, name="Potential")

devsim.node_model(device=device, region=region, name="n", equation="ni * exp(Potential / Vt)")

devsim.node_model(device=device, region=region, name="p", equation="ni * exp(-Potential / Vt)")

for c in ("top", "bot"):

devsim.contact_node_model(device=device, contact=c, name=f"{c}_bc",

equation="Potential - Vt * log(NetDoping / ni)")

devsim.contact_node_model(device=device, contact=c, name=f"{c}_bc:Potential", equation="1")

devsim.contact_equation(device=device, contact=c, name="PotentialEquation", node_model=f"{c}_bc")

# --- 5. Initial Guess and Solve ---

devsim.node_model(device=device, region=region, name="Equil", equation="Vt * log(NetDoping / ni)")

vals = devsim.get_node_model_values(device=device, region=region, name="Equil")

devsim.set_node_values(device=device, region=region, name="Potential", values=vals)

# --- 6. Excess Carrier Models ---

devsim.node_model(device=device, region=region, name="n0", equation="ni * exp(Vt * log(NetDoping / ni) / Vt)")

devsim.node_model(device=device, region=region, name="p0", equation="ni * exp(-Vt * log(NetDoping / ni) / Vt)")

devsim.node_model(device=device, region=region, name="delta_n", equation="n - n0")

devsim.node_model(device=device, region=region, name="delta_p", equation="p - p0")

devsim.write_devices(file="part1_resistor_final.vtu", type="vtk")

print("SUCCESS: Resistor profile generated according to Fonstad 6.2.")

# Part2

code ;

import devsim

device = "Resistor"

region = "Silicon"

# --- 1. Mesh Creation (0 to 50um with 100 node points) ---

# Total length = 50e-4 cm. Spacing = 5e-6 cm provides 100 points.

devsim.create_1d_mesh(mesh="mesh1")

devsim.add_1d_mesh_line(mesh="mesh1", pos=0.0, ps=5e-6, tag="top")

devsim.add_1d_mesh_line(mesh="mesh1", pos=50e-4, ps=5e-6, tag="bot")

devsim.add_1d_contact(mesh="mesh1", name="top", tag="top", material="metal")

devsim.add_1d_contact(mesh="mesh1", name="bot", tag="bot", material="metal")

devsim.add_1d_region(mesh="mesh1", material="Si", region=region, tag1="top", tag2="bot")

devsim.finalize_mesh(mesh="mesh1")

devsim.create_device(mesh="mesh1", device=device)

# --- 2. Physical Parameters (Silicon) ---

ni = 1.0e10

Vt = 0.02585

c1 = 1.0e17        # Donor concentration (Constant for uniform profile)

c2 = 0.0           # Acceptor concentration (Constant for uniform profile)

devsim.set_parameter(device=device, region=region, name="ni", value=ni)

devsim.set_parameter(device=device, region=region, name="Vt", value=Vt)

# --- 3. Net Doping Profile (Corrected per Professor's Advice) ---

# Formula: NetDoping = c1*Ndn - c2*Nap. Using uniform Ndn=1 and Nap=0.

devsim.node_model(device=device, region=region, name="NDn", equation=f"{c1}")

devsim.node_model(device=device, region=region, name="NAp", equation=f"{c2}")

devsim.node_model(device=device, region=region, name="NetDoping", equation="NDn - NAp")

# --- 4. Potential and Carrier Models ---

devsim.node_solution(device=device, region=region, name="Potential")

devsim.node_model(device=device, region=region, name="n", equation="ni * exp(Potential / Vt)")

devsim.node_model(device=device, region=region, name="p", equation="ni * exp(-Potential / Vt)")

# --- 5. Boundary Conditions (Zero Bias Equilibrium) ---

for c in ("top", "bot"):

# Ohmic contact boundary condition at zero bias

devsim.contact_node_model(device=device, contact=c, name=f"{c}_bc",

equation="Potential - Vt * log(NetDoping / ni)")

devsim.contact_node_model(device=device, contact=c, name=f"{c}_bc:Potential", equation="1")

devsim.contact_equation(device=device, contact=c, name="PotentialEquation", node_model=f"{c}_bc")

# --- 6. The Equilibrium Solve ---

# Set initial guess based on uniform doping log-ratio

devsim.node_model(device=device, region=region, name="Equil", equation="Vt * log(NetDoping / ni)")

vals = devsim.get_node_model_values(device=device, region=region, name="Equil")

devsim.set_node_values(device=device, region=region, name="Potential", values=vals)

# --- 7. Excess Carrier Models (Verification) ---

# n0 and p0 are equilibrium concentrations. delta_n should be 0 at zero bias.

devsim.node_model(device=device, region=region, name="n0", equation="ni * exp(Vt * log(NetDoping / ni) / Vt)")

devsim.node_model(device=device, region=region, name="p0", equation="ni * exp(-Vt * log(NetDoping / ni) / Vt)")

devsim.node_model(device=device, region=region, name="delta_n", equation="n - n0")

devsim.node_model(device=device, region=region, name="delta_p", equation="p - p0")

devsim.write_devices(file="part2_zero_bias_uniform.vtu", type="vtk")

print("SUCCESS: Zero bias simulation complete for uniform resistor.")

# Part3

import devsim

import math

device = "Resistor"

region = "Silicon"

# --- 1. Mesh Construction ---

devsim.create_1d_mesh(mesh="mesh1")

devsim.add_1d_mesh_line(mesh="mesh1", pos=0.0, ps=5e-6, tag="top")

devsim.add_1d_mesh_line(mesh="mesh1", pos=50e-4, ps=5e-6, tag="bot")

devsim.add_1d_contact(mesh="mesh1", name="top", tag="top", material="metal")

devsim.add_1d_contact(mesh="mesh1", name="bot", tag="bot", material="metal")

devsim.add_1d_region(mesh="mesh1", material="Si", region=region, tag1="top", tag2="bot")

devsim.finalize_mesh(mesh="mesh1")

devsim.create_device(mesh="mesh1", device=device)

# --- 2. Parameters (Fonstad Fundamentals) ---

ni, Vt, c1 = 1.0e10, 0.02585, 1.0e17

eps_si, q_val = 1.03594e-12, 1.602e-19

phi_equil = Vt * math.log(c1 / ni)

devsim.set_parameter(name="ni", value=ni)

devsim.set_parameter(name="Vt", value=Vt)

devsim.set_parameter(name="eps_si", value=eps_si)

devsim.set_parameter(name="q", value=q_val)

# --- 3. Solution Variable & Initialization ---

devsim.node_solution(device=device, region=region, name="Potential")

# Initialize with equilibrium potential

devsim.set_node_values(device=device, region=region, name="Potential", values=[phi_equil] * 1001)

devsim.edge_from_node_model(device=device, region=region, node_model="Potential")

# --- 4. Physics Models ---

devsim.node_model(device=device, region=region, name="NetDoping", equation=f"{c1}")

devsim.node_model(device=device, region=region, name="n", equation="ni * exp(Potential / Vt)")

devsim.node_model(device=device, region=region, name="p", equation="ni * exp(-Potential / Vt)")

devsim.node_model(device=device, region=region, name="ChargeDensity", equation="q * (p - n + NetDoping)")

# Jacobian derivative - CRITICAL to stop "Singular Matrix"

devsim.node_model(device=device, region=region, name="ChargeDensity:Potential",

equation="-q * (ni/Vt * exp(Potential/Vt) + ni/Vt * exp(-Potential/Vt))")

devsim.edge_model(device=device, region=region, name="ElectricField",

equation="(Potential@n0 - Potential@n1)*EdgeInverseLength")

devsim.edge_model(device=device, region=region, name="PotentialEdgeFlux",

equation="eps_si * ElectricField")

# --- 5. Potential Equation ---

devsim.equation(device=device, region=region, name="PotentialEquation",

variable_name="Potential", node_model="ChargeDensity",

edge_model="PotentialEdgeFlux", variable_update="positive")

# --- 6. Boundary Conditions (0.3V Bias) ---

# Top Contact

devsim.contact_node_model(device=device, contact="top", name="top_bc", equation=f"Potential - {phi_equil + 0.3}")

devsim.contact_node_model(device=device, contact="top", name="top_bc:Potential", equation="1")

devsim.contact_equation(device=device, contact="top", name="PotentialEquation", node_model="top_bc")

# Bot Contact

devsim.contact_node_model(device=device, contact="bot", name="bot_bc", equation=f"Potential - {phi_equil}")

devsim.contact_node_model(device=device, contact="bot", name="bot_bc:Potential", equation="1")

devsim.contact_equation(device=device, contact="bot", name="PotentialEquation", node_model="bot_bc")

# --- 7. Solve ---

devsim.solve(type="dc", solver_type="direct", absolute_error=1e-12, relative_error=1e-12, maximum_iterations=50)

# --- 8. Post-Processing ---

devsim.node_model(device=device, region=region, name="delta_n", equation="n - NetDoping")

devsim.write_devices(file="part3_final.vtu", type="vtk")

print("SUCCESS: Result saved to part3_final.vtu")

# Part4

import devsim

import math

device = "Diode"

region = "Silicon"

# --- 1. Mesh ---

devsim.create_1d_mesh(mesh="mesh1")

devsim.add_1d_mesh_line(mesh="mesh1", pos=0.0, ps=1e-5, tag="left")

devsim.add_1d_mesh_line(mesh="mesh1", pos=50e-4, ps=1e-5, tag="right")

devsim.add_1d_contact(mesh="mesh1", name="anode", tag="left", material="metal")

devsim.add_1d_contact(mesh="mesh1", name="cathode", tag="right", material="metal")

devsim.add_1d_region(mesh="mesh1", material="Si", region=region, tag1="left", tag2="right")

devsim.finalize_mesh(mesh="mesh1")

devsim.create_device(mesh="mesh1", device=device)

# --- 2. Parameters ---

ni, Vt, conc = 1.0e10, 0.02585, 1.0e17

eps_si, q_val = 1.03594e-12, 1.602e-19

phi_p = -Vt * math.log(conc / ni)

phi_n = Vt * math.log(conc / ni)

devsim.set_parameter(name="ni", value=ni)

devsim.set_parameter(name="Vt", value=Vt)

devsim.set_parameter(name="eps_si", value=eps_si)

devsim.set_parameter(name="q", value=q_val)

# --- 3. Step Function Doping ---

devsim.node_model(device=device, region=region, name="NetDoping",

equation=f"ifelse(x < 25e-4, -{conc}, {conc})")

# --- 4. Initialization (Crucial to prevent Overflow) ---

devsim.node_solution(device=device, region=region, name="Potential")

x_coords = devsim.get_node_model_values(device=device, region=region, name="x")

# Hard-code the equilibrium potentials to prevent the solver from guessing wildly

init_pot = [phi_p if x < 25e-4 else phi_n for x in x_coords]

devsim.set_node_values(device=device, region=region, name="Potential", values=init_pot)

devsim.edge_from_node_model(device=device, region=region, node_model="Potential")

# --- 5. Physics Models ---

devsim.node_model(device=device, region=region, name="n", equation="ni * exp(Potential / Vt)")

devsim.node_model(device=device, region=region, name="p", equation="ni * exp(-Potential / Vt)")

devsim.node_model(device=device, region=region, name="ChargeDensity", equation="q * (p - n + NetDoping)")

# Jacobian Derivative - Mandatory

devsim.node_model(device=device, region=region, name="ChargeDensity:Potential",

equation="-q * (ni/Vt * exp(Potential/Vt) + ni/Vt * exp(-Potential/Vt))")

devsim.edge_model(device=device, region=region, name="ElectricField",

equation="(Potential@n0 - Potential@n1)*EdgeInverseLength")

devsim.edge_model(device=device, region=region, name="PotentialEdgeFlux",

equation="eps_si * ElectricField")

# --- 6. Equation (FIX: Use log_damp to prevent exponential overflow) ---

devsim.equation(device=device, region=region, name="PotentialEquation",

variable_name="Potential", node_model="ChargeDensity",

edge_model="PotentialEdgeFlux", variable_update="log_damp")

# --- 7. Boundary Conditions ---

for contact, pot in [("anode", phi_p), ("cathode", phi_n)]:

devsim.contact_node_model(device=device, contact=contact, name=f"{contact}_bc", equation=f"Potential - {pot}")

devsim.contact_node_model(device=device, contact=contact, name=f"{contact}_bc:Potential", equation="1")

devsim.contact_equation(device=device, contact=contact, name="PotentialEquation", node_model=f"{contact}_bc")

# --- 8. Solve ---

devsim.solve(type="dc", solver_type="direct", absolute_error=1e-12, relative_error=1e-12, maximum_iterations=100)

# --- 9. Output ---

devsim.write_devices(file="part4_diode.vtu", type="vtk")

print("SUCCESS: PN Junction result saved to part4_diode.vtu")

# Part5

import devsim

import math

device = "Diode"

region = "Silicon"

# --- 1. Mesh Construction ---

devsim.create_1d_mesh(mesh="mesh1")

devsim.add_1d_mesh_line(mesh="mesh1", pos=0.0, ps=1e-5, tag="left")

devsim.add_1d_mesh_line(mesh="mesh1", pos=50e-4, ps=1e-5, tag="right")

devsim.add_1d_contact(mesh="mesh1", name="anode", tag="left", material="metal")

devsim.add_1d_contact(mesh="mesh1", name="cathode", tag="right", material="metal")

devsim.add_1d_region(mesh="mesh1", material="Si", region=region, tag1="left", tag2="right")

devsim.finalize_mesh(mesh="mesh1")

devsim.create_device(mesh="mesh1", device=device)

# --- 2. Parameters ---

ni, Vt, conc = 1.0e10, 0.02585, 1.0e17

eps_si, q_val = 1.03594e-12, 1.602e-19

phi_p = -Vt * math.log(conc / ni)

phi_n = Vt * math.log(conc / ni)

devsim.set_parameter(name="ni", value=ni)

devsim.set_parameter(name="Vt", value=Vt)

devsim.set_parameter(name="eps_si", value=eps_si)

devsim.set_parameter(name="q", value=q_val)

# --- 3. Step Function Doping ---

devsim.node_model(device=device, region=region, name="NetDoping",

equation=f"ifelse(x < 25e-4, -{conc}, {conc})")

# --- 4. Initialization ---

devsim.node_solution(device=device, region=region, name="Potential")

x_coords = devsim.get_node_model_values(device=device, region=region, name="x")

init_pot = [phi_p if x < 25e-4 else phi_n for x in x_coords]

devsim.set_node_values(device=device, region=region, name="Potential", values=init_pot)

devsim.edge_from_node_model(device=device, region=region, node_model="Potential")

# --- 5. Physics Models ---

devsim.node_model(device=device, region=region, name="n", equation="ni * exp(Potential / Vt)")

devsim.node_model(device=device, region=region, name="p", equation="ni * exp(-Potential / Vt)")

devsim.node_model(device=device, region=region, name="ChargeDensity", equation="q * (p - n + NetDoping)")

devsim.node_model(device=device, region=region, name="ChargeDensity:Potential",

equation="-q * (ni/Vt * exp(Potential/Vt) + ni/Vt * exp(-Potential/Vt))")

devsim.edge_model(device=device, region=region, name="ElectricField",

equation="(Potential@n0 - Potential@n1)*EdgeInverseLength")

devsim.edge_model(device=device, region=region, name="PotentialEdgeFlux",

equation="eps_si * ElectricField")

# --- 6. Equation ---

devsim.equation(device=device, region=region, name="PotentialEquation",

variable_name="Potential", node_model="ChargeDensity",

edge_model="PotentialEdgeFlux", variable_update="log_damp")

# --- 7. Initial Boundary Conditions (Equilibrium) ---

for contact, pot in [("anode", phi_p), ("cathode", phi_n)]:

devsim.contact_node_model(device=device, contact=contact, name=f"{contact}_bc", equation=f"Potential - {pot}")

devsim.contact_node_model(device=device, contact=contact, name=f"{contact}_bc:Potential", equation="1")

devsim.contact_equation(device=device, contact=contact, name="PotentialEquation", node_model=f"{contact}_bc")

# --- 8. Bias Sweep Loop ---

# Voltages: 0, Forward (0.1, 0.3, 0.5), Reverse (-0.1, -0.3, -0.5)

bias_list = [0.0, 0.1, 0.3, 0.5, -0.1, -0.3, -0.5]

print("\n--- Starting PN Junction Bias Sweep ---")

print("Bias (V) | Status")

print("------------------")

for v in bias_list:

# Update Anode Potential: Equilibrium Potential + Bias

devsim.contact_node_model(device=device, contact="anode", name="anode_bc",

equation=f"Potential - ({phi_p + v})")

try:

devsim.solve(type="dc", solver_type="direct", absolute_error=1e-12,

relative_error=1e-12, maximum_iterations=50)

# Calculate Excess Carrier Concentration: delta_n = n(v) - n(equilibrium)

# Note: We define n_intrinsic_p as n at equilibrium on p-side for delta_n analysis

devsim.node_model(device=device, region=region, name="delta_n",

equation=f"n - (ni * exp(({phi_p if v >= 0 else phi_p})/Vt))")

# Save VTU file for each bias

fname = f"diode_{v}V.vtu".replace("-", "neg")

devsim.write_devices(file=fname, type="vtk")

print(f"{v:7.1f}  | Success -> {fname}")

except Exception as e:

print(f"{v:7.1f}  | Failed: {e}")

print("------------------")

print("All files saved. Process complete.")

I AM FACING SOME TROUBLE IN MY PART 6 AND 7 CODE , CURRENTY I AM TRAVELLING TO CAMPUS I WILL COMLETE PART 6 AND 7 ONCE I REACH CAMPUS. SORRY FOR THE TROUBLE.